SETTING UP THE SCANNERS IN MASTER - SLAVE MODE

Firstly, the scanners should always be used in an upright position This is, with the Mirror, DMX Connectors & Dip Switches at the top, and the mains lead at the bottom. Failure to ensure this will cause lamp failure and premature wearing of the Servo Motors and is not covered by the Warranty

To set up the scanners to operate in Master - Slave mode, first designate one unit to be the "Master", or control unit. This is usually the first scanner in the chain. It is worth marking this unit, with a coloured label, or similar, in order to indentify it as the Master unit for future use. On this unit, locate the "dip" switches, these are a row of switches located on the top of each unit, next to the XLR connection Sockets. On the Master unit, ensure that the following  switches are moved down into the "ON" position:

DIP Switches: 1,2,3,4,5,6,10,11 - move to the ON position.

This will now identify this unit as the Control unit, and will run through it's 6 built in programs automatically, controlled by the sound picked up by the built-in Microphone, and synchronising all other scanners that are connected to it.

Next ensure that the DIP switches of every Scanner that will be controlled from this unit (SLAVES) are in the OFF position (Factory Default position).

Unpack the supplied connecting lead(s). Position the Master unit so that it is the first unit in the chain. Connect the Lead to the DMX OUT socket on the Master unit, and plug the end of this lead into the second unit in the chain, by plugging into the DMX IN socket on the second unit. Next plug, another lead from the DMX OUT socket on the second unit to the DMX IN socket on the third unit. Continue this until all of your Scanners are connected to the Main Master unit, in a chain. The Master unit is capable of suppling upto 16 other Slave scanners connected to it.

If you wish to use each scanner in it's "stand alone" mode - i.e running independantly to each other, then omit the connecting cables, and ensure that the DIP switches: 1,2,3,4,5,6,10,11 are in the ON position.

When first switched on the Scanners do not initiate immediately, each unit will run through a self-test which may take upto 40 seconds. Some Mechanical noise may be heard during this period - This is Normal.

IF THE UNITS DON'T SYNCHRONISE

(1) - Ensure that the DIP Switches 1,2,3,4,5,6,10,11 are all set to the ON position on the MASTER unit

(2) - Ensure that the DIP Switches on each connected SLAVE unit are all set to the OFF position

(3) - Check that each unit in the chain is connected firmly to the DMX OUT socket of the previous unit

(4) - Check that Power is applied to Each Scanner and that there is Audio Present for the Master Unit

.If the Slave units appear to lose Syncronisation on occasion, or cease to follow the DMX signal of the Master unit then a DMX terminator plug may be required. This is a plug in adaptor which simply plugs into the final unit in the chain and terminates the DMX signal. This is rare, but If you experience this type of problem, we will send you a Terminator plug free of charge.